Name of the Experiment:

Combinational and Sequential Circuit Implementation Using iVerilog.

Theory:

Decoder: Decoder is a combinational circuit that has ‘n’ input lines and maximum of 2^n output lines. One of these outputs will be active High based on the combination of inputs present, when the decoder is enabled. That means decoder detects a particular code.

Truth table of a 2 to 4 decoder is given below:

D-Flip Flop with Synchronous Enable: A D flip-flop, or Data flip-flop, is a type of digital storage element used in sequential logic circuits. It captures the value of the D input (data) at a specific portion of the clock cycle (usually the rising or falling edge of the clock signal) and holds that value until the next clock edge. A D flip-flop with a synchronous enable (often referred to as a D flip-flop with an enable or D flip-flop with a clock enable) is an enhanced version of the standard D flip-flop. It includes an additional input, the enable signal (EN), which controls whether the flip-flop should update its state on the clock edge or maintain its current state.

Source Code:

Decoder:

HDL Code:

module decoder2to4

(

input [1:0] A,

input E,

output [3:0] Y

);

assign Y[3] = E & A[1] & A[0];

assign Y[2] = E & A[1] & ~A[0];

assign Y[1] = E & ~A[1] & A[0];

assign Y[0] = E & ~A[1] & ~A[0];

endmodule

Testbench:

`timescale 1ns/1ps

module decoder2to4_tb;

reg [1:0] A;

reg E;

wire [3:0] Y;

decoder2to4 uut

(

.A(A),

.E(E),

.Y(Y)

);

initial begin

$dumpfile("test.vcd");

$dumpvars(0,decoder2to4_tb);

A[1] = 0;

A[0] = 0;

E = 0;

#20;

E = 1;

#20;

A[0] = 1;

#20;

A[1] = 1;

#20;

A[0] = 0;

#20;

end

initial begin

$monitor("E=%d A[1]=%d A[0]=%d Y[3]=%d Y[2]=%d Y[1]=%d Y[0]=%d\n",E, A[1], A[0], Y[3], Y[2], Y[1],Y[0]);

end

endmodule

D Flip Flop:

HDL Code:

module d_ff

(

input wire clk,

input wire reset,

input wire en,

input wire d,

output reg q

);

always @(posedge clk, posedge reset)

begin

if (reset)

q <= 1'b0;

else if (en)

q <= d;

end

endmodule

Testbench:

`timescale 1ns/1ns

module d_ff_tb;

reg clk;

reg reset;

reg en;

reg d;

wire q;

d_ff circuit1

(clk, reset, en, d, q);

always begin

clk = ~clk;

#10;

end

initial begin

$dumpfile("test.vcd");

$dumpvars(0, d_ff_tb);

clk <= 0;

reset <= 1;

en <= 0;

d <= 0;

#20;

reset <= 0;

en <= 1;

d <= 1;

#20;

$finish;

end

initial begin

$monitor("clk = %b, reset = %b, en = %b, d = %b, q = %b", clk, reset, en, d, q);

end

endmodule

Output:

Decoder:

Terminal Output:

GTKWave output:

D-Flip Flop:

Terminal Output:

GTKWave: